Deployment Manual

Permafrost–Infrastructure Settlement Prediction

Retrospective Prediction Experiment Framework

Geo-FiLM ST-GAT Validation on Public Permafrost Data

Version 5.0 — August 2026

Raymond Xiaohu Fan fanxiaohu@whcp.edu.cn

1. Overview

1.1 Purpose

This project implements a retrospective prediction framework for permafrost–infrastructure settlement estimation. The core innovation is the Geo-FiLM ST-GAT model, which injects geological priors (Active Layer Thickness, soil–structure coefficient, and thawing index) into a spatio-temporal graph attention network via Feature-wise Linear Modulation (FiLM) conditioning. The framework benchmarks eight models across four categories to validate the effectiveness of physics-informed machine learning for cryospheric hazard prediction.

1.2 Key Features

Geo-FiLM conditioning: Geological priors modulate ST-GAT feature representations, enabling physics-aware spatial–temporal prediction.

Multi-model comparison: Eight models spanning physics-informed GNN, standard GNN, PINN, pure physics, and traditional ML — providing comprehensive ablation evidence.

Physical constraint validation: Stefan equation–derived violation rate quantifies physical plausibility of each model's predictions.

Flexible data pipeline: Supports public real data (TPDC/PANGAEA), local cached data, and simulation fallback — ensuring the framework runs under any circumstances.

AutoDL cloud deployment: Pre-configured shell script for GPU cloud instances with automatic environment setup, data download, and dimension validation.

1.3 Eight-Model Comparison Matrix

| # | Model | Category | Role in Comparison |
| --- | --- | --- | --- |
| 1 | Geo-FiLM ST-GAT | Physics-Informed GNN | Core innovation — validates FiLM conditioning |
| 2 | Standard ST-GAT | GNN | Ablation — proves FiLM improves prediction |
| 3 | ST-GCN | GNN | Baseline — graph convolution + temporal convolution |
| 4 | DCRNN | GNN | Baseline — diffusion convolution RNN |
| 5 | PINN-Permafrost | Physics-Informed ML | Baseline — Stefan equation as physics constraint |
| 6 | Stefan-Only | Pure Physics | Baseline — pure Stefan equation prediction |
| 7 | Random Forest | Traditional ML | Baseline — sklearn ensemble |
| 8 | XGBoost | Traditional ML | Baseline — gradient boosting |

2. System Requirements

2.1 Hardware

| Component | Minimum | Recommended |
| --- | --- | --- |
| GPU | NVIDIA GPU with ≥ 16 GB VRAM | RTX 3090 / A100 / V100 |
| RAM | 8 GB | 16 GB or more |
| Storage (data disk) | 50 GB free space | 100 GB or more |
| CPU | 4 cores | 8+ cores |

2.2 Software

| Package | Version | Notes |
| --- | --- | --- |
| Python | 3.10+ | 3.12 tested on AutoDL RTX4090 |
| PyTorch | 2.0+ | CUDA 12.4 runtime on AutoDL; pre-installed on cloud GPU instances |
| NumPy | 1.24+ | Core array operations |
| pandas | 2.0+ | Data handling |
| scikit-learn | 1.3+ | Random Forest baseline |
| XGBoost | 2.0+ | Gradient boosting baseline |
| matplotlib | 3.7+ | Visualization (optional) |
| PyYAML | 6.0+ | Configuration file parsing |
| scipy | 1.11+ | Scientific computing support |

Note: torch_geometric is NOT required. All GNN models (ST-GAT, ST-GCN, DCRNN) are hand-implemented to avoid dependency conflicts and version incompatibilities.

3. Project Structure

| Path | Description |
| --- | --- |
| run_main.py | Main entry point — orchestrates config loading, data acquisition, preprocessing, model training, and result output |
| setup_autodl.sh | AutoDL cloud deployment script — GPU check, dependency install, data download, dimension validation, and fallback experiment |
| configs/ | YAML configuration files (simulation_fallback.yaml, etc.) |
| models/ | Model implementations: geo_film_st_gat.py, st_gat.py, stgcn.py, dcrnn.py, pinn_permafrost.py, stefan_model.py |
| baselines/ | ML baselines: rf_baseline.py (Random Forest), gbrt_baseline.py (XGBoost) |
| trainers/ | trainer.py — generic training pipeline, run_all_experiments() |
| data/ | Data pipeline: synthetic_generator.py, preprocess.py, download_data.py |
| datasets/raw/ | Raw data files from public sources (original format) |
| datasets/processed/ | Preprocessed .npz data files ready for model training |
| evaluation/ | metrics.py (RMSE, MAE, R², etc.), physical_constraint.py (Stefan validation) |
| outputs/ | Result output directory (txt, csv, diagnostics, log) |

4. Data Acquisition

4.1 Data Format Specification

All datasets are preprocessed into a unified .npz (NumPy compressed archive) format. Each .npz file contains the following arrays:

| Key | Shape | Description |
| --- | --- | --- |
| X_train | (N_train, T_in, F) | Training feature time series, F = 12 features |
| X_test | (N_test, T_in, F) | Test feature time series |
| y_train | (N_train, T_out) | Training settlement labels (future T_out steps) |
| y_test | (N_test, T_out) | Test settlement labels |
| adj_matrix | (N_total, N_total) | Spatial adjacency matrix (graph structure) |
| geo_features | (N_total, G) | FiLM conditioning priors, G = 3 dimensions |
| feature_names | list[str] | Names of the 12 input features |
| node_ids | (N_total) | Node identifiers |
| metadata | dict | Dataset metadata (name, time range, source, etc.) |

4.2 Feature Dimensions (F = 12)

| Index | Feature Name | Unit | Description |
| --- | --- | --- | --- |
| 0 | cumulative_deformation | mm | InSAR cumulative deformation |
| 1 | deformation_velocity | mm/year | Deformation rate |
| 2 | coherence | 0–1 | InSAR coherence |
| 3 | ground_temp | °C | Ground temperature |
| 4 | thawing_index (I_t) | °C·day | Thawing degree-day index |
| 5 | freezing_index | °C·day | Freezing degree-day index |
| 6 | moisture | % | Soil moisture content |
| 7 | elevation | m | Altitude |
| 8 | slope_angle | ° | Terrain slope |
| 9 | permafrost_type | code | Permafrost classification code |
| 10 | infrastructure_type | code | Infrastructure type code |
| 11 | seasonal_amplitude | mm | Seasonal deformation amplitude |

4.3 FiLM Conditioning Priors (G = 3)

| Index | Feature Name | Description |
| --- | --- | --- |
| 0 | ALT | Active Layer Thickness derived from Stefan equation (m) |
| 1 | k_s | Soil–structure coefficient (dimensionless) |
| 2 | I_t | Thawing index (°C·day) — Stefan equation input |

4.4 Data Sources

The framework supports three tiers of data acquisition, following a strict priority order:

4.4.1 Tier 1 — Public Real Data (Primary)

| ID | Dataset | Source | DOI / Link |
| --- | --- | --- | --- |
| D1 | Qinghai–Tibet Corridor InSAR 2D Deformation (2017–2022) | TPDC | 10.11888/cryos.tpdc.300400 |
| D2 | GTN-P CALM Active Layer Thickness | PANGAEA | 10.1594/PANGAEA.972777 |
| D3 | Qinghai–Tibet Plateau Integrated Monitoring | TPDC | 789e838e-16ac-4539-bb7e |

TPDC (National Tibetan Plateau Data Center) provides free access upon registration. PANGAEA datasets are open-access. Both sources are recognized by reviewers in cryospheric research.

4.4.2 Tier 2 — Alternative Regional Data

| ID | Dataset | Source | Priority |
| --- | --- | --- | --- |
| A1 | Qilian Mountain Chaimu Railway Deformation + Disease Data | TPDC | Alt-1 |
| A2 | Qinghai–Tibet Corridor Permafrost–Engineering Hydrothermal Observation | ESSD | Alt-2 |
| A5 | Northeast Permafrost Engineering Foundation Stability | NCDC | Alt-3 |
| A8 | Yeniugou Permafrost Deformation + ALT | TPDC | Alt-4 |

4.4.3 Tier 3 — Simulation Fallback (Always Available)

When no real data is accessible, the framework generates physically realistic simulation data via a Stefan equation–driven generator (SyntheticDataGenerator). The simulation produces InSAR deformation, ALT, and settlement data calibrated to Qilian Mountain / Qaidam Basin permafrost conditions.

Warning: Simulation data is suitable for framework validation and debugging, but should NOT be used for publication results. Replace with real public data for final experiments.

4.5 Data Acquisition Priority

Cached local data (if .npz file exists in datasets/processed/) — fastest, no download

Public data download (TPDC/PANGAEA) — requires registration/authentication

Simulation fallback (automatic) — always available as safety net

5. Installation and Setup

5.1 Quick Setup via Shell Script (Recommended)

For AutoDL or similar GPU cloud platforms, use the provided deployment script which handles environment verification, dependency installation, data download, and validation in a single step:

cd /root/retrospective_prediction
bash setup_autodl.sh

The script performs the following steps automatically:

System information check: GPU model, CUDA version, Python version, disk capacity

Python dependency installation: numpy, pandas, scikit-learn, matplotlib, pyyaml, xgboost, scipy. PyTorch is NOT re-installed on cloud instances (pre-installed by platform).

Data download and preprocessing: attempts public data sources, falls back to simulation

Feature dimension validation: prints actual F and G dimensions, warns if mismatched with config

Simulation fallback experiment: runs a quick experiment to verify the entire pipeline works

5.2 Manual Installation

If the shell script fails or for local development environments:

pip install numpy pandas scikit-learn matplotlib pyyaml xgboost scipy

Note: PyTorch must be installed separately with CUDA support. On cloud GPU instances, PyTorch is typically pre-installed. For local setups, follow the official PyTorch installation guide for your CUDA version.

Verify all dependencies are installed:

python3 -c "import torch, numpy, sklearn, xgboost; print('All OK!')"

5.3 AutoDL Cloud Platform Specifics

When deploying on AutoDL (or similar GPU cloud platforms), the following conventions apply:

| Setting | Value | Rationale |
| --- | --- | --- |
| DATA_ROOT | /root/autodl-tmp | Large-capacity data disk (/dev/md0 mount point); system disk /root has limited space (~50 GB) |
| Code location | /root/retrospective_prediction | Can reside on system disk — code is small |
| Data/Output location | /root/autodl-tmp/datasets/ and /root/autodl-tmp/outputs/ | Must be on data disk — datasets and results are large |
| GPU detection | Automatic | torch.cuda.is_available() → cuda or cpu |

5.4 DATA_ROOT Resolution Priority

The system resolves the data storage root directory by the following priority:

CLI argument --data-root (explicit override, highest priority)

Environment variable DATA_ROOT

Auto-detection: /root/autodl-tmp exists and is writable (AutoDL cloud)

Default: project root directory (local development / sandbox)

6. Running Experiments

6.1 Basic Usage

Run the default simulation fallback experiment:

python3 run_main.py

Or specify a configuration file explicitly:

python3 run_main.py --config configs/simulation_fallback.yaml

Run with real public data (if available):

python3 run_main.py --config configs/real_data.yaml

6.2 Command-Line Arguments

| Argument | Type | Default | Description |
| --- | --- | --- | --- |
| --config | str | configs/simulation_fallback.yaml | YAML configuration file path |
| --skip-download | flag | False | Skip data download, use local cached data |
| --skip-train | flag | False | Skip training, only data generation + dimension check |
| --epochs | int | None | Override training epoch count (for debugging) |
| --device | str | None | Force device: cpu / cuda / cuda:0 |
| --data-root | str | None | Override data/output storage root directory |
| --verbose / -v | flag | False | Verbose level 1: print key progress messages |
| --debug / -d | flag | False | Verbose level 2: print detailed shapes, tensor info |
| --quiet / -q | flag | False | Verbose level 0: minimal output (only final results) |
| --log-file | nargs | default | Save all output to DATA_ROOT/outputs/run_log.txt |
| --no-log | flag | False | Disable log file, print only to console |

6.3 Typical Usage Patterns

6.3.1 Quick Validation (Simulation)

python3 run_main.py --config configs/simulation_fallback.yaml --epochs 50

Runs a shortened simulation experiment to verify the pipeline works end-to-end.

6.3.2 Full Experiment (Real Data)

python3 run_main.py --config configs/real_data.yaml --verbose

Full training with real data and progress messages.

6.3.3 Debug Mode

python3 run_main.py --debug --epochs 20 --log-file

Maximum verbosity with tensor shape details and log file output.

6.3.4 Silent Batch Mode

python3 run_main.py --quiet --no-log

Minimal output for automated/batch execution scenarios.

6.4 Critical Runtime Safety Mechanisms

The framework incorporates several safety mechanisms derived from real-world deployment experience on GPU cloud platforms:

Dynamic input_dim override: input_dim is NOT hardcoded in config. It is dynamically set from X_train.shape[2] at runtime, overriding any YAML value. This prevents dimension mismatches between config and actual data.

Dynamic geo_dim override: Similarly, geo_dim is set from geo_features.shape[1] at runtime.

YAML type safety: All numeric values from YAML are explicitly cast with float()/int() to prevent TypeError when PyYAML parses values like 1e-5 as strings.

GPU auto-detection: Device is automatically selected (cuda if available, cpu otherwise). Models and data are consistently moved to the same device.

Data source fallback: If public data download fails, the framework automatically generates simulation data as a fallback — the experiment always runs.

7. Configuration

7.1 YAML Configuration Parameters

The experiment behavior is controlled via YAML configuration files.

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| dataset_name | str | simulation_qilian | Dataset identifier (used for .npz filename) |
| region | str | Qilian Mountain | Geographic region label |
| data_source | str | simulation | Data source: "simulation", "tpdc", "pangaea", etc. |
| hidden_dim | int | 64 | Hidden dimension for GNN models |
| num_heads | int | 4 | Number of attention heads in ST-GAT |
| num_spatial_layers | int | 2 | Number of spatial GAT layers |
| dropout | float | 0.1 | Dropout rate |
| epochs | int | 200 | Training epochs (can be overridden by --epochs CLI arg) |
| lr | float | 0.001 | Learning rate |
| lambda_phys | float | 0.3 | Physics loss weight for PINN models |
| batch_size | int | 32 | Batch size for training |
| output_dim | int | 1 | Output dimension (settlement prediction horizon) |
| normalization_method | str | zscore | Feature normalization: "zscore" or "minmax" |
| train_ratio | float | 0.8 | Train/test split ratio |
| T_in | int | 24 | Input time window length |
| T_out | int | 1 | Output prediction horizon |
| k_thaw | float | 1.2 | Thermal conductivity for Stefan equation (W/m·K) |
| L_latent | float | 3.34e7 | Volumetric latent heat for Stefan equation (J/m³) |

Note: input_dim and geo_dim are NOT configurable — they are dynamically determined from the actual data shapes at runtime. Never hardcode these values in YAML files.

7.2 Simulation Data Parameters

When using simulation fallback, the following additional parameters control synthetic data generation:

| Parameter | Default | Description |
| --- | --- | --- |
| N_nodes | 50 | Number of simulated monitoring nodes |
| T_steps | 24 | Number of time steps in simulation |
| feature_dim | 12 | Number of features per node per time step |
| geo_dim | 3 | Number of geological prior dimensions |
| noise_level | 0.05 | Gaussian noise level added to settlement labels |

8. Output Files

All output files are stored in the outputs/ directory under DATA_ROOT.

8.1 Output Directory Structure

DATA_ROOT/outputs/
├── experiment_results.txt    — Human-readable results table
├── experiment_results.csv    — CSV format results
├── diagnostics.txt           — Per-model diagnostic details
└── run_log.txt               — Full experiment log (optional)

8.2 Output File Descriptions

| File | Format | Description |
| --- | --- | --- |
| experiment_results.txt | Plain text | Human-readable results table including dataset info, configuration, data source, dimensions, and per-model metrics. Contains a warning banner if results are based on simulation data. |
| experiment_results.csv | CSV | Machine-parseable results with columns: model, rmse, mae, r2, phys_violation_rate, film_improvement, train_time_sec, eval_time_sec, status |
| diagnostics.txt | Plain text | Detailed per-model diagnostic information: input shapes, training curve summary, device info, and any error messages |
| run_log.txt | Plain text | Complete experiment log with all console output. Generated when --log-file flag is used (default). |

8.3 CSV Column Definitions

| Column | Type | Description |
| --- | --- | --- |
| model | str | Model name (e.g., "Geo-FiLM ST-GAT") |
| rmse | float | Root Mean Square Error (m) |
| mae | float | Mean Absolute Error (m) |
| r2 | float | Coefficient of Determination (R²) |
| phys_violation_rate | float | Percentage of predictions violating Stefan constraint (%) |
| film_improvement | float | RMSE reduction of Geo-FiLM vs Standard ST-GAT (%) |
| train_time_sec | float | Training wall-clock time (seconds) |
| eval_time_sec | float | Evaluation wall-clock time (seconds) |
| status | str | Experiment status: "success" or error description |

9. Evaluation Metrics

Each model is evaluated on the following metrics, which are designed to assess both prediction accuracy and physical plausibility:

| Metric | Unit | Description | Relevance |
| --- | --- | --- | --- |
| RMSE | m | Root Mean Square Error of settlement predictions | Standard accuracy measure; directly comparable across models |
| MAE | m | Mean Absolute Error of settlement predictions | Complementary to RMSE; less sensitive to outliers |
| R² | 0–1 | Coefficient of Determination | Proportion of variance explained by the model |
| Physical Violation Rate | % | Fraction of predictions where ¦y_pred¦ > k_s × ALT | Measures physical plausibility against Stefan equation constraints |
| FiLM Improvement | % | RMSE reduction: (RMSE_std − RMSE_film) / RMSE_std × 100 | Quantifies the benefit of FiLM conditioning (ablation evidence) |

9.1 Stefan Equation Physical Constraint

The physical constraint validation is based on the Stefan equation for Active Layer Thickness (ALT):

ALT = sqrt(2 × k_thaw × I_t × 86400 / L)

Where k_thaw is thermal conductivity (W/m·K), I_t is thawing index (°C·day), and L is volumetric latent heat (J/m³). A prediction violates the physical constraint if the absolute predicted settlement exceeds k_s × ALT, where k_s is the soil–structure coefficient.

10. Model Architecture Summary

10.1 Geo-FiLM ST-GAT (Core Innovation)

The Geo-FiLM ST-GAT model integrates geological priors into a spatio-temporal graph attention network through Feature-wise Linear Modulation (FiLM). The FiLM layer generates scaling (γ) and shifting (β) parameters from geological features (ALT, k_s, I_t), which modulate the ST-GAT hidden representations:

γ = W_γ · geo_features + b_γ
β = W_β · geo_features + b_β
h_modulated = γ × h_spatial + β

This enables the network to adapt its spatial–temporal processing based on site-specific geological conditions, producing physics-aware settlement predictions.

10.2 Standard ST-GAT (Ablation Baseline)

Identical architecture to Geo-FiLM ST-GAT but without the FiLM conditioning layer. Serves as the ablation baseline to demonstrate that geological priors improve prediction accuracy.

10.3 Other Models

| Model | Architecture | Key Design |
| --- | --- | --- |
| ST-GCN | Graph Convolution + Temporal Convolution | Spectral graph filtering with ChebConv; temporal 1D convolution |
| DCRNN | Diffusion Convolution + GRU | Random-walk diffusion on graph; recurrent temporal processing |
| PINN-Permafrost | Neural Network + Stefan Loss | Standard MLP with Stefan equation constraint added to training loss |
| Stefan-Only | Pure Stefan Equation | Direct ALT computation from Stefan formula as settlement prediction — no learning component |
| Random Forest | sklearn Ensemble | Bagging of decision trees on flat feature vectors |
| XGBoost | Gradient Boosting | Boosted trees on flat feature vectors with regularization |

11. Data Download from Remote Server

After running experiments on a GPU cloud instance, result files can be downloaded to your local machine using the following methods:

11.1 SCP (Secure Copy)

Download individual files:

scp -P <PORT> root@<HOST>:/root/autodl-tmp/outputs/experiment_results.txt ~/Downloads/
scp -P <PORT> root@<HOST>:/root/autodl-tmp/outputs/experiment_results.csv ~/Downloads/

Download the entire outputs directory:

scp -P <PORT> -r root@<HOST>:/root/autodl-tmp/outputs ~/Downloads/autodl_outputs/

Replace <PORT> and <HOST> with the SSH connection information provided by your cloud platform. For AutoDL instances, these values are found on the instance details page.

11.2 Cloud Platform File Manager

Most GPU cloud platforms (AutoDL, etc.) provide a web-based file manager. Navigate to /root/autodl-tmp/outputs/ and click the download button next to each file.

11.3 HTTP Server Method

Start a temporary HTTP server on the remote instance:

cd /root/autodl-tmp/outputs
python3 -m http.server 8080

Then access files via browser at http://<PUBLIC_IP>:8080/. This method requires the instance to have port mapping configured.

12. Troubleshooting

12.1 Common Issues

| Issue | Cause | Solution |
| --- | --- | --- |
| TypeError in math.log(delta) | YAML parses 1e-5 as string | All config numeric values are auto-cast with float()/int() in run_main.py |
| Dimension mismatch: input_dim | Config hardcoded F ≠ actual data F | input_dim is dynamically overridden from X_train.shape[2] at runtime |
| Dimension mismatch: geo_dim | Config hardcoded G ≠ actual data G | geo_dim is dynamically overridden from geo_features.shape[1] at runtime |
| CUDA device mismatch | Global model on CPU, eval data on GPU | All models and data are consistently moved to the detected device |
| Single class in y_train | Preprocessed data has only 1 label value | run_main.py checks num_classes ≥ 2 and raises ValueError if violated |
| Public data download fails | TPDC/PANGAEA requires registration | Framework automatically falls back to simulation data generation |
| GPU not detected | CUDA driver/runtime mismatch | Check nvidia-smi output; setup_autodl.sh prints CUDA compatibility info |

12.2 Verbose Levels for Debugging

Use the following verbosity levels to diagnose issues:

--quiet (level 0): Print only final results. Suitable for batch execution.

--verbose (level 1): Default. Print key progress messages, dimension info, and per-model metrics.

--debug (level 2): Print detailed tensor shapes, model internals, and per-step diagnostic information. Recommended for troubleshooting.

Combine with --log-file to capture all output for offline analysis:

python3 run_main.py --debug --log-file

13. Reference

13.1 Stefan Equation

The Stefan equation models the thickness of the active layer (the seasonally thawing layer above permafrost) as a function of thermal conductivity and thawing index:

ALT = sqrt(2 × k_thaw × I_t × 86400 / L)

Parameters: k_thaw = 1.2 W/(m·K) (default thawing-period thermal conductivity), L = 3.34 × 10⁷ J/m³ (volumetric latent heat of water→ice phase change), I_t in °C·day (thawing degree-day index).

13.2 FiLM Conditioning

Feature-wise Linear Modulation (FiLM) is a general conditioning mechanism that applies affine transformations to neural network feature maps based on conditioning information. In this framework, geological priors (ALT, k_s, I_t) generate per-node scaling and shifting parameters that modulate the ST-GAT spatial features, enabling site-specific physics-aware prediction.

13.3 Data Source References

TPDC — National Tibetan Plateau Data Center: https://data.tpdc.ac.cn/

PANGAEA — Data Publisher for Earth & Environmental Science: https://www.pangaea.de/

ESSD — Earth System Science Data journal: https://www.earth-system-science-data.net/